Newly Built App Board Testing Procedure

Written by Hunter Mills on 5/29

Based on Bootload/VM instructions for the now defunct Dev Board

Bootloader Instructions

- Disconnect the Jumper, and flip the VM switch toward the 6-pin connector

- Plug the App Board into the Bootload/VM Board

- Plug the Bootload/VM Board ‘s Boot cable into the App Board

- Plug the Bootload/VM Board’s USB into the computer

- Load assembler.jar in the Bootloading Folder (Should be within Folder AppTest)

  - a java terminal should appear with a text box and two buttons labeled "asm" and "download"

- Type in "new-chip"

- This text should appear

  - enter-bsl

  - passwd v-erase

  - print r1 65502 4288 64

  - se [0 0] v-timints

  - ok

- Within this textbox click in "enter-bsl" and hit enter

  - The system should respond "ok"

- Select "passwd v-erase" and hit and enter

  - If it says "passwd FF", STOP, Something is wrong, check power, and swich locations.  If you still get FF then stop and check solder connections on Dev-Board

  - if it says "passwd 90", then continue

- Select" print r1 4288 64" and hit enter

  - The system should respond with a bunch of integers between 0 and 255

  - If the system responds with all -1s, do not continue

- Select "w1 65502 se [0 0] v-timints" and hit enter

  - The system should respond "w1 FFDE 90"

- Click the "asm" button

  - The system should respond "368 bytes"

- Click the "download" button

  - The system should respond w1 FFDE 90, w1 FE00 90, w1 FE40 90, w1 FE80 90, w1 FEC0 90, w1 FF00 90, w1 FF40 90

- Unplug the USB and the Bootloading Cable from the App Board

- This Completes the Bootloading Process

VM Instructions

- Disconnect the Jumper, and flip the VM switch toward the 6-pin connector

- Plug the App Board into the Bootload/VM Board

- Plug the Bootload/VM Board ‘s VM cable into the App Board

- Plug the Bootload/VM Board’s USB into the computer

- While in terminal execute the following steps

  - Enter the appropriate VM directory (normally the newest)

  - Run jl.sh

  - Type and Return "send $ff print recc"

    - This should be responded with and integer value of 36

  - Type and Return "asm"

  - Type and Return "download"

    - Lights should blink

  - Type and Return "dump $1000 0"

  - Type and Return "erase-seg $1000" -- Be Very Careful on This Step

  - Type and Return "wfl $1000 [ 16 ]"

  - Type and Return "dump $1000 0"

    - This should respond with the first entry as 16 (decimal) or 10 (hex) followed by multiple FFs

  - Hit ctrl + c

- Unplug the Bootload/VM Board’s USB into the computer

- This completes loading the VM

App Board Function Test

- Connect the Jumper, and flip the VM switch away from the 6-pin connector

- Connect the new App Board to the Tester Board

- Enter the “ulogo” folder

  - Run “jl.sh”

  - A terminal should open

  - Run the following command “.init-newboard”

    - This should compile and download the test code to the Board

  - Run the following command “.test-board”

  - Follow the onscreen instructions. This includes:

    - All pins are turned on and read by the tester (should return all 1’s)

    - All pins are turned off and read by the tester (should return all 0’s)

    - The on Board tri-color LED is tested by cycling through red blue and green

    - The DACs are tested by emulating a sine wave routing the signal to an Oscilloscope and test

- If there are any failures in the above procedure, check solder connections and test again.

- This completes testing the App Board. At this point the board is ready to use and should be cataloged.